//Copyright 1986-2020 Xilinx, Inc. All Rights Reserved.

//--------------------------------------------------------------------------------

//Tool Version: Vivado v.2020.2 (win64) Build 3064766 Wed Nov 18 09:12:45 MST 2020

//Date        : Tue Dec 23 18:52:52 2025

//Host        : LAPTOP-U67BPUSH running 64-bit major release  (build 9200)

//Command     : generate_target WLJC_wrapper.bd

//Design      : WLJC_wrapper

//Purpose     : IP block netlist

//--------------------------------------------------------------------------------

`timescale 1 ps / 1 ps

module WLJC_wrapper

(DDR_addr,

DDR_ba,

DDR_cas_n,

DDR_ck_n,

DDR_ck_p,

DDR_cke,

DDR_cs_n,

DDR_dm,

DDR_dq,

DDR_dqs_n,

DDR_dqs_p,

DDR_odt,

DDR_ras_n,

DDR_reset_n,

DDR_we_n,

FIXED_IO_ddr_vrn,

FIXED_IO_ddr_vrp,

FIXED_IO_mio,

FIXED_IO_ps_clk,

FIXED_IO_ps_porb,

FIXED_IO_ps_srstb,

adc_clk_out_0,

adc_data_0,

adc_otr_0,

clk_in1_0,

dac_clk_out_0,

dac_data_0,

dac_wrt_out_0,

resetn_0,

wr_key);

inout [14:0]DDR_addr;

inout [2:0]DDR_ba;

inout DDR_cas_n;

inout DDR_ck_n;

inout DDR_ck_p;

inout DDR_cke;

inout DDR_cs_n;

inout [3:0]DDR_dm;

inout [31:0]DDR_dq;

inout [3:0]DDR_dqs_n;

inout [3:0]DDR_dqs_p;

inout DDR_odt;

inout DDR_ras_n;

inout DDR_reset_n;

inout DDR_we_n;

inout FIXED_IO_ddr_vrn;

inout FIXED_IO_ddr_vrp;

inout [53:0]FIXED_IO_mio;

inout FIXED_IO_ps_clk;

inout FIXED_IO_ps_porb;

inout FIXED_IO_ps_srstb;

output adc_clk_out_0;

input [11:0]adc_data_0;

input adc_otr_0;

input clk_in1_0;

output dac_clk_out_0;

output [13:0]dac_data_0;

output dac_wrt_out_0;

input resetn_0;

input wr_key;

wire [14:0]DDR_addr;

wire [2:0]DDR_ba;

wire DDR_cas_n;

wire DDR_ck_n;

wire DDR_ck_p;

wire DDR_cke;

wire DDR_cs_n;

wire [3:0]DDR_dm;

wire [31:0]DDR_dq;

wire [3:0]DDR_dqs_n;

wire [3:0]DDR_dqs_p;

wire DDR_odt;

wire DDR_ras_n;

wire DDR_reset_n;

wire DDR_we_n;

wire FIXED_IO_ddr_vrn;

wire FIXED_IO_ddr_vrp;

wire [53:0]FIXED_IO_mio;

wire FIXED_IO_ps_clk;

wire FIXED_IO_ps_porb;

wire FIXED_IO_ps_srstb;

wire adc_clk_out_0;

wire [11:0]adc_data_0;

wire adc_otr_0;

wire clk_in1_0;

wire dac_clk_out_0;

wire [13:0]dac_data_0;

wire dac_wrt_out_0;

wire resetn_0;

wire wr_key;

WLJC WLJC_i

(.DDR_addr(DDR_addr),

.DDR_ba(DDR_ba),

.DDR_cas_n(DDR_cas_n),

.DDR_ck_n(DDR_ck_n),

.DDR_ck_p(DDR_ck_p),

.DDR_cke(DDR_cke),

.DDR_cs_n(DDR_cs_n),

.DDR_dm(DDR_dm),

.DDR_dq(DDR_dq),

.DDR_dqs_n(DDR_dqs_n),

.DDR_dqs_p(DDR_dqs_p),

.DDR_odt(DDR_odt),

.DDR_ras_n(DDR_ras_n),

.DDR_reset_n(DDR_reset_n),

.DDR_we_n(DDR_we_n),

.FIXED_IO_ddr_vrn(FIXED_IO_ddr_vrn),

.FIXED_IO_ddr_vrp(FIXED_IO_ddr_vrp),

.FIXED_IO_mio(FIXED_IO_mio),

.FIXED_IO_ps_clk(FIXED_IO_ps_clk),

.FIXED_IO_ps_porb(FIXED_IO_ps_porb),

.FIXED_IO_ps_srstb(FIXED_IO_ps_srstb),

.adc_clk_out_0(adc_clk_out_0),

.adc_data_0(adc_data_0),

.adc_otr_0(adc_otr_0),

.clk_in1_0(clk_in1_0),

.dac_clk_out_0(dac_clk_out_0),

.dac_data_0(dac_data_0),

.dac_wrt_out_0(dac_wrt_out_0),

.resetn_0(resetn_0),

.wr_key(wr_key));

endmodule

module adc_capture (

input wire adc_clk,             // ADC时钟

input wire locked,              // 时钟锁定信号

input wire [11:0] adc_data,     // 12位ADC数据输入

input wire adc_otr,             // ADC超量程标志

output reg [11:0] adc_captured  // 修改：输出改回 12位

);

// 输入同步寄存器（保持12位）

reg [11:0] adc_data_sync1, adc_data_sync2;

reg adc_otr_sync1, adc_otr_sync2;

always @(posedge adc_clk) begin

if (!locked) begin

// 时钟未锁定时，复位所有寄存器

adc_data_sync1 <= 12'd0;

adc_data_sync2 <= 12'd0;

adc_otr_sync1  <= 1'b0;

adc_otr_sync2  <= 1'b0;

adc_captured   <= 12'd0;  // 修改：复位为12位0

end else begin

// 三级同步提高稳定性（保持不变）

adc_data_sync1 <= adc_data;

adc_data_sync2 <= adc_data_sync1;

adc_otr_sync1  <= adc_otr;

adc_otr_sync2  <= adc_otr_sync1;

if (adc_otr_sync2) begin

// 超量程时输出12位全1 (0xFFF)

adc_captured <= 12'hFFF;  // 修改：12位最大值

end else begin

// 正常数据：直接输出原始12位，不补零

adc_captured <= adc_data_sync2;  // 修改：直接赋值

end

end

end

Endmodule

module dac_controller (

input  wire        clk,           // 100 MHz

input  wire        dac_clk,       // 20 MHz

input  wire        locked,        // 时钟锁定信号（高有效，来自Clocking Wiz）

input  wire [13:0] dac_data_in,   // 输入数据（始终有效）

output wire        dac_clk_out,

output wire        dac_wrt_out,

output reg  [13:0] dac_data       // DAC输出数据

);

assign dac_clk_out = dac_clk;

assign dac_wrt_out = dac_clk;

/*-------------- 100 MHz 域：数据同步 --------------*/

reg [13:0] data_sync;

always @(posedge clk) begin

data_sync <= dac_data_in;

end

/*-------------- 跨时钟域同步到20MHz --------------*/

reg [13:0] data_cdc_meta, data_cdc_sync;

always @(posedge dac_clk) begin

data_cdc_meta <= data_sync;

data_cdc_sync <= data_cdc_meta;

end

/*-------------- locked信号同步到20MHz域 --------------*/

// 对异步locked信号进行二级同步，防止亚稳态

reg locked_meta, locked_sync;

always @(posedge dac_clk) begin

locked_meta <= locked;

locked_sync <= locked_meta;

end

/*-------------- 20MHz域：DAC输出更新 --------------*/

always @(posedge dac_clk) begin

if (!locked_sync) begin

dac_data <= 14'd0;  // 时钟未锁定时强制输出0

end else begin

dac_data <= data_cdc_sync;  // 正常输出数据

end

end

Endmodule

// adc_fifo_minimal_safe.v

// 修改记录：输入改为12位，内部自动扩展为32位以适配DMA

module adc_fifo_minimal_safe (

input  wire        wr_clk,

input  wire [11:0] din,          // <--- 修改：输入改为 12 位

input  wire        wr_en,

input  wire        rd_clk,

output wire [31:0] dout,         // 输出保持 32 位 (给DMA)

input  wire        rd_en,

output wire        empty

);

// ============================================================================

// 参数定义

// ============================================================================

localparam DEPTH = 8192;

localparam ADDR_WIDTH = 13;

// ============================================================================

// 内部信号声明

// ============================================================================

wire        full_internal;

wire [ADDR_WIDTH-1:0] wr_addr;

wire [ADDR_WIDTH-1:0] rd_addr;

// ============================================================================

// RAM存储体 (保持32位宽，适配DMA对齐)

// ============================================================================

reg [31:0] mem [0:DEPTH-1];     // 8K × 32位

// ============================================================================

// 写操作：12位 -> 32位扩展

// ============================================================================

reg [ADDR_WIDTH:0] wr_ptr_bin = 0;

assign wr_addr = wr_ptr_bin[ADDR_WIDTH-1:0];

always @(posedge wr_clk) begin

if (wr_en && !full_internal) begin

// <--- 修改关键点：

// 目标32位 = 20位补零 + 12位数据

mem[wr_addr] <= {20'b0, din};

wr_ptr_bin <= wr_ptr_bin + 1;

end

end

// ============================================================================

// 读操作 (保持不变，直接输出32位)

// 注意：这种写法属于 FWFT (First Word Fall Through) 模式，

// 数据在 rd_en 拉高之前就已经准备好在 dout 上了。

// 这非常适合配合之前提供的 fifo_to_axis 模块使用。

// ============================================================================

reg [ADDR_WIDTH:0] rd_ptr_bin = 0;

assign rd_addr = rd_ptr_bin[ADDR_WIDTH-1:0];

always @(posedge rd_clk) begin

if (rd_en && !empty) begin

rd_ptr_bin <= rd_ptr_bin + 1;

end

end

assign dout = mem[rd_addr];  // 直接输出32位

// ============================================================================

// 格雷码同步 (保持不变)

// ============================================================================

function [ADDR_WIDTH:0] bin2gray(input [ADDR_WIDTH:0] bin);

bin2gray = bin ^ (bin >> 1);

endfunction

reg [ADDR_WIDTH:0] wr_ptr_gray;

reg [ADDR_WIDTH:0] rd_ptr_gray;

reg [ADDR_WIDTH:0] wr_ptr_gray_sync1, wr_ptr_gray_sync2;

reg [ADDR_WIDTH:0] rd_ptr_gray_sync1, rd_ptr_gray_sync2;

always @(posedge wr_clk) wr_ptr_gray <= bin2gray(wr_ptr_bin);

always @(posedge rd_clk) begin

wr_ptr_gray_sync1 <= wr_ptr_gray;

wr_ptr_gray_sync2 <= wr_ptr_gray_sync1;

end

always @(posedge rd_clk) rd_ptr_gray <= bin2gray(rd_ptr_bin);

always @(posedge wr_clk) begin

rd_ptr_gray_sync1 <= rd_ptr_gray;

rd_ptr_gray_sync2 <= rd_ptr_gray_sync1;

end

// ============================================================================

// 空满判断 (保持不变)

// ============================================================================

wire [ADDR_WIDTH:0] wr_ptr_gray_rdclk = wr_ptr_gray_sync2;

wire [ADDR_WIDTH:0] rd_ptr_gray_wrclk = rd_ptr_gray_sync2;

assign empty = (bin2gray(rd_ptr_bin) == wr_ptr_gray_rdclk);

assign full_internal = (bin2gray(wr_ptr_bin) ==

{~rd_ptr_gray_wrclk[ADDR_WIDTH:ADDR_WIDTH-1],

rd_ptr_gray_wrclk[ADDR_WIDTH-2:0]});

Endmodule

module data_buffer_async (

input wire wr_clk,

input wire rd_clk,

input wire rst_n,

input wire [11:0] adc_data,

input wire wr_en,

input wire rd_en,

output reg [11:0] adc_out_data,

output wire empty,

output wire full

);

parameter FIFO_DEPTH = 1024;

parameter ADDR_WIDTH = 10;

// FIFO存储器

(* ram_style = "block" *) reg [11:0] fifo_memory [0:FIFO_DEPTH-1];

// 读写指针

reg [ADDR_WIDTH:0] wr_ptr, rd_ptr;  // 多一位用于空满判断

wire [ADDR_WIDTH-1:0] wr_addr, rd_addr;

assign wr_addr = wr_ptr[ADDR_WIDTH-1:0];

assign rd_addr = rd_ptr[ADDR_WIDTH-1:0];

// 空满标志

assign empty = (wr_ptr == rd_ptr);

assign full = (wr_ptr[ADDR_WIDTH] != rd_ptr[ADDR_WIDTH]) &&

(wr_ptr[ADDR_WIDTH-1:0] == rd_ptr[ADDR_WIDTH-1:0]);

// 写指针逻辑

always @(posedge wr_clk or negedge rst_n) begin

if (!rst_n) begin

wr_ptr <= 0;

end else if (wr_en && !full) begin

fifo_memory[wr_addr] <= adc_data;

wr_ptr <= wr_ptr + 1'b1;

end

end

// 读指针逻辑

always @(posedge rd_clk or negedge rst_n) begin

if (!rst_n) begin

rd_ptr <= 0;

adc_out_data <= 12'd0;

end else if (rd_en && !empty) begin

adc_out_data <= fifo_memory[rd_addr];

rd_ptr <= rd_ptr + 1'b1;

end

end

Endmodule

module adc_wr_toggle (

input  wire adc_clk,       // 1MHz (注意：对于消抖，1MHz有点慢，但也能用)

input  wire locked,

input  wire key_n,

output reg  wr_en

);

// 1MHz时钟下，20ms = 20,000个周期

localparam CNT_MAX = 17'd20000;

reg [16:0] debounce_cnt = 0;

reg key_stable = 1'b1;

reg key_prev = 1'b1;

reg key_synced = 1'b1; // 用于同步外部按键信号

// 1. 先把异步按键信号同步到 adc_clk 域，防止亚稳态

always @(posedge adc_clk) begin

key_synced <= key_n;

end

// 2. 正确的消抖逻辑

always @(posedge adc_clk) begin

if (key_synced == key_stable) begin

debounce_cnt <= 0; // 状态一致，计数器清零

end else begin

// 状态不一致，开始计数

if (debounce_cnt < CNT_MAX) begin

debounce_cnt <= debounce_cnt + 1;

end else begin

// 只有计满20ms，才认为状态真的变了

key_stable <= key_synced;

debounce_cnt <= 0;

end

end

end

// 3. 边沿检测与翻转

wire key_pressed = (key_prev == 1'b1) && (key_stable == 1'b0); // 下降沿

always @(posedge adc_clk) begin

key_prev <= key_stable;

if (!locked) begin

wr_en <= 1'b0;

end else if (key_pressed) begin

wr_en <= ~wr_en;

end

end

Endmodule

module pulse_signal_generator (

input wire clk,                 // 100MHz时钟

input wire clk_locked,          // 时钟锁定信号（来自Clocking Wiz），高有效

input wire [15:0] pulse_width,  // 脉冲宽度（时钟周期数）

input wire [15:0] pulse_period, // 脉冲周期（时钟周期数）

input wire [7:0] pulse_amplitude, // 脉冲幅度控制（8位）

output reg [13:0] dds_output    // 14位DDS输出

);

// 内部信号定义

reg [15:0] period_counter;         // 周期计数器

reg [15:0] pulse_counter;          // 脉冲宽度计数器

reg state;                         // 状态机状态（1位足够）

wire [13:0] amplitude_scaled;      // 幅度缩放后的输出

// 状态定义

localparam PULSE_HIGH  = 1'b0;

localparam PULSE_LOW   = 1'b1;

// 幅度缩放：将8位幅度转换为14位DAC值

assign amplitude_scaled = {pulse_amplitude, 6'b0}; // 左移6位，近似满量程

// 状态转移逻辑：增加时钟锁定控制

always @(posedge clk) begin

if (!clk_locked) begin

// 时钟未锁定：强制复位状态机，输出清零

state <= PULSE_LOW;

period_counter <= 16'd0;

pulse_counter <= 16'd0;

dds_output <= 14'd0;

end else begin

// 时钟锁定：正常工作

case (state)

PULSE_HIGH: begin

// 输出脉冲高电平

dds_output <= amplitude_scaled;

if (pulse_counter < pulse_width - 1) begin

pulse_counter <= pulse_counter + 1'b1;

end else begin

state <= PULSE_LOW;

pulse_counter <= 16'd0;

period_counter <= 16'd0;

end

end

PULSE_LOW: begin

// 输出低电平

dds_output <= 14'd0;

if (period_counter < (pulse_period - pulse_width - 1)) begin

period_counter <= period_counter + 1'b1;

end else begin

state <= PULSE_HIGH;

period_counter <= 16'd0;

pulse_counter <= 16'd0;

end

end

default: state <= PULSE_LOW;

endcase

end

end

Endmodule

# 时钟约束

set_property PACKAGE_PIN U18 [get_ports clk_in1_0]

set_property IOSTANDARD LVCMOS33 [get_ports clk_in1_0]

# 复位约束

set_property PACKAGE_PIN N15 [get_ports resetn_0]

set_property IOSTANDARD LVCMOS33 [get_ports resetn_0]

set_property PULLUP true [get_ports resetn_0]

# DAC (AN9767 通道1) 接口约束

set_property PACKAGE_PIN J19 [get_ports dac_wrt_out_0]      ;# P1_WRT  仅新增这一行

set_property IOSTANDARD LVCMOS33 [get_ports dac_wrt_out_0]

set_property SLEW FAST [get_ports dac_wrt_out_0]

set_property DRIVE 8   [get_ports dac_wrt_out_0]

set_property PACKAGE_PIN K19 [get_ports dac_clk_out_0]      ;# P1_CLK  原已存在

set_property IOSTANDARD LVCMOS33 [get_ports dac_clk_out_0]

set_property SLEW FAST [get_ports dac_clk_out_0]

set_property DRIVE 8 [get_ports dac_clk_out_0]

set_property IOSTANDARD LVCMOS33 [get_ports {dac_data_0[*]}]

set_property PACKAGE_PIN K17 [get_ports {dac_data_0[0]}]

set_property PACKAGE_PIN K18 [get_ports {dac_data_0[1]}]

set_property PACKAGE_PIN M19 [get_ports {dac_data_0[2]}]

set_property PACKAGE_PIN M20 [get_ports {dac_data_0[3]}]

set_property PACKAGE_PIN L19 [get_ports {dac_data_0[4]}]

set_property PACKAGE_PIN L20 [get_ports {dac_data_0[5]}]

set_property PACKAGE_PIN J18 [get_ports {dac_data_0[6]}]

set_property PACKAGE_PIN H18 [get_ports {dac_data_0[7]}]

set_property PACKAGE_PIN G19 [get_ports {dac_data_0[8]}]

set_property PACKAGE_PIN G20 [get_ports {dac_data_0[9]}]

set_property PACKAGE_PIN F19 [get_ports {dac_data_0[10]}]

set_property PACKAGE_PIN F20 [get_ports {dac_data_0[11]}]

set_property PACKAGE_PIN F16 [get_ports {dac_data_0[12]}]

set_property PACKAGE_PIN F17 [get_ports {dac_data_0[13]}]

set_property SLEW FAST [get_ports {dac_data_0[*]}]

set_property DRIVE 8 [get_ports {dac_data_0[*]}]

# ADC (AN9238) 接口约束

set_property PACKAGE_PIN W19 [get_ports adc_clk_out_0]

set_property IOSTANDARD LVCMOS33 [get_ports adc_clk_out_0]

set_property SLEW FAST [get_ports adc_clk_out_0]

set_property IOSTANDARD LVCMOS33 [get_ports {adc_data_0[*]}]

set_property PACKAGE_PIN W18 [get_ports {adc_data_0[0]}]

set_property PACKAGE_PIN R14 [get_ports {adc_data_0[1]}]

set_property PACKAGE_PIN P14 [get_ports {adc_data_0[2]}]

set_property PACKAGE_PIN Y17 [get_ports {adc_data_0[3]}]

set_property PACKAGE_PIN Y16 [get_ports {adc_data_0[4]}]

set_property PACKAGE_PIN W15 [get_ports {adc_data_0[5]}]

set_property PACKAGE_PIN V15 [get_ports {adc_data_0[6]}]

set_property PACKAGE_PIN Y14 [get_ports {adc_data_0[7]}]

set_property PACKAGE_PIN W14 [get_ports {adc_data_0[8]}]

set_property PACKAGE_PIN P18 [get_ports {adc_data_0[9]}]

set_property PACKAGE_PIN N17 [get_ports {adc_data_0[10]}]

set_property PACKAGE_PIN U15 [get_ports {adc_data_0[11]}]

set_property PULLDOWN true [get_ports {adc_data_0[*]}]

set_property PACKAGE_PIN U14 [get_ports adc_otr_0]

set_property IOSTANDARD LVCMOS33 [get_ports adc_otr_0]

set_property PULLDOWN true [get_ports adc_otr_0]

# 控制信号约束

set_property PACKAGE_PIN N16 [get_ports wr_key]

set_property IOSTANDARD LVCMOS33 [get_ports wr_key]

set_property PULLUP true [get_ports wr_key]

// clk_div_10.v

// 功能：将输入时钟 10 分频 (10MHz -> 1MHz)

module clk_div_10 (

input  wire clk_in,   // 来自 IP 核的 10MHz

input  wire rst_n,    // 复位信号 (可用 locked)

output reg  clk_out   // 输出给 ADC 的 1MHz

);

reg [2:0] cnt; // 0~4 计数器

always @(posedge clk_in or negedge rst_n) begin

if (!rst_n) begin

cnt <= 3'd0;

clk_out <= 1'b0;

end else begin

if (cnt >= 3'd4) begin  // 0~4 共5个周期翻转一次

cnt <= 3'd0;

clk_out <= ~clk_out; // 翻转，实现 10 分频

end else begin

cnt <= cnt + 1;

end

end

end

Endmodule

module fifo_to_axis (

input wire clk,

input wire rst_n,

// === 连接 FIFO ===

output wire        fifo_rd_en,

input  wire [31:0] fifo_dout,

input  wire        fifo_empty,

// === 连接 DMA ===

output wire [31:0] m_axis_tdata,

output wire        m_axis_tvalid,

output wire        m_axis_tlast,

output wire [3:0]  m_axis_tkeep,

input  wire        m_axis_tready

);

// ============================================================

// 修改 1: 包长度改为 1,000,000 (对应 1秒 @ 1MHz)

// ============================================================

localparam PACKET_LEN = 1000000;

// ============================================================

// 修改 2: 计数器位宽增加到 32位 (足以容纳 1,000,000)

// ============================================================

reg [31:0] transfer_cnt;

// ============================================================

// 逻辑实现 (保持不变)

// ============================================================

wire axis_handshake;

assign axis_handshake = m_axis_tvalid & m_axis_tready;

assign m_axis_tvalid = !fifo_empty;

assign m_axis_tdata  = fifo_dout;

assign m_axis_tkeep  = 4'b1111;

assign fifo_rd_en    = axis_handshake;

// 生成 TLAST

assign m_axis_tlast = (transfer_cnt == PACKET_LEN - 1) && axis_handshake;

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

transfer_cnt <= 0;

end else begin

if (axis_handshake) begin

if (transfer_cnt == PACKET_LEN - 1) begin

transfer_cnt <= 0;

end else begin

transfer_cnt <= transfer_cnt + 1;

end

end

end

end

Endmodule